The full title of the report

Electronic project Report

Full name of the student

Rajesh Jasti

Auckland University Technology

Month, year of submission

Contents

Introduction	1

Half Wave Bridge Dual Rail Power Supply	2

Analog and Digital Piano Project	3

Analog Piano	4

Digital Piano	5

Practical Session	7

1.	Half Wave Bridge Dual Rail Power Supply Project	7

2.	2. Analog and Digital Piano Project	13

Conclusion	14

References	15

# Introduction

The Project Piano is produced by connecting a variable frequency NE555 timer to a speaker. The piano is a wonderful musical instrument and using the NE555D timer IC chip and ATtiny2313A microcontroller. By allowing the timer to produce 8 different kinds of sounds, this concept of the piano is obtained. By enabling it to run as square wave mode or using delay function, the various sounds are generated via a timer, with a variable frequency output option. The entire project is separated into two parts, project 1 and project 2, Starting with project 1 designing the power supply for the piano by choosing various electronic components. Designed circuit bring it to reality unique electronic component, Altium designer software equipped with power full circuit design futures incorporate with schematic designing futures and PCB designing. By following electrical diagrams created schematic diagrams with the right components choosing from libraries. The same has finished by applying schematic rules with errors free and documented. The next part is creating a PCB contains a flat sheet of insulating materials lawyers of copper coil, based on requirements PCB can design with two layers or multilayers, two layers board has copper on both sides, multi lawyer boards sandwich additional copper lawyers of insulating materials, lawyers connected with “Vias”, Two lawyers are generally assigned to supply and ground, other two lawyers are used for signal routing between two components. After cutting PCB as requirements, the schematic is transferred to PCB layout, this is a crucial part of placing components on PCB by following schematic aim is to reduce routing length between components for better power dissipations. To reduce manufacturing errors and better durability, setting up the PCB rules is very important especially voltage, high voltage, ground, and signal, avoiding 90 degrees routing, using 45 degrees angle will give a better routing structure. By completing the error-free rule check will be finished PCB designed process. Next process designed PCB will be sent to the manufacturer, after manufacturing PCB board electronic parts assembled on board, through-hole components soldered by using a traditional soldering iron, surface mount components soldered using a special machine to solder. After the assembly process before connecting the power supply, it is good practice to check the board for short circuits. Project-2 is the same process as above but 2nd part of design combinations of analog and digital components can produce different frequencies outputs to buzzer when press button pressed when jumper switch to the digital mode by pressing S9 button outputs different frequencies same as a piano.

# Half Wave Bridge Dual Rail Power Supply

The power supply is a critical part of the project, many electronic circuits or devices fail with poor design of the power supply, good power supply determines systems reliability and saves system parts. T1 AC Input connected to half-wave bridge dual-rail consists of diodes D1, D3 each cycle of alternating current diode conducts only half of the cycle block current another half cycle alternating current. By doing that it converts alternating input power into direct current output power, capacitors C1, C3 are used for reducing ripple of the output, resistors R4, R5 placed in the circuit to reduce surge current when the device is turned on sometimes used as a bleeder. U1 liner voltage regulator used to maintain stable voltage, variable input voltage result in stable constant output voltage, U1 receives +12V from the rectifier it outputs +5V, C3, C4, C5 and C6 are input, output decoupling capacitors for regulator they provide low impedance at high frequency, reduces noise from the source and load, to avoid bounces from the regulated voltages to achieve stable output, the capacitor can also be required for frequency compensation of the regulator IC. D4, D5 are Schottky diodes used for low turs on voltage, low loss energy at higher frequencies, capable of rectifying a current by facilitating quick transition conducting to blocking state, D4 allows the voltage to pass through D5 block voltage, D4 gets voltage from regulator output and it passes to T4 output. R1 is resister placed in the restricts flow of current to glow LED, D2 is diode when turs on power supply D2 lights up.

T1 AC Input going into the circuit

F1 Fuse sued for protecting the circuit from the high current

D1 and D3 are Diodes positive half-cycle to the alternating current passed through, negative half cycle restricts passing through the diodes

D4, D5 are Schottky diodes used for low turs on voltage, low loss energy at higher frequencies, capable of rectifying a current by facilitating quick transition conducting to blocking state, D4 allows the voltage to pass through D5 block voltage.

R4, R5 is placed in the circuit to reduce surge current when the device is turned on some times used as a bleeder. U1 liner voltage regulator used to maintain stable voltage, variable input voltage result in stable constant output voltage.

C1, C2 Capacitors are used for reducing the ripple of the output.

# Analog and Digital Piano Project

Project-2 is a functional part, receives power supply from the power supply circuit previously designed as ad described above, this circuit involved ATtiny2313A microcontroller which has input outputs, the timer is connected to the microcontroller which generates PWM or delay function to execute the digital function for piano, when S9 is pressed microcontroller outputs different frequencies to buzzer based on coding uploaded into the microcontroller EPROM, apart from that various electronic components are coupled to MCU such as a buzzer, piano switches, jumper, P1, P2 connectors are a source of power supply to circuit +12V and +5V, Schottky diodes are connected to 5v and 12v supply to restrict the reverse flow of current. U1 timer powered with 12v supply, S1 to 58 switches are connected to resister R1 to R8 parallel, part of Analog output, resister is used different values to produce different frequencies, when the user pressed switches from S1 to S8 it produces different frequencies, timer outputs to the buzzer to produce piano effects. P3, P4 are grounding connectors to the circuit, P6 is jumper using it for switching from Analog to digital, S9 switch is equipped with pull-up resister need to be activated during embedded coding development. When the user pressed S9 to switch its input to the microcontroller, MCU outputs resister R10 to D3 emitting diode, at the same time its buzzer produces different music.

D1, D2 are Schottky diodes it restricts the reverse flow of current

C1, C5 are coupling capacitors it reduces noises

Analog part of the Circuit S1 to S8 connected to resisters from R1 to R8 with different values of resistances, when the user is pressed switches frequencies produces based on fixed resistance values, fixed frequencies can't be changed unless hard were parts changed.

Digital part of the circuit S9 when the user pressed switch based on the program developed and uploaded into EPROM, the program executes and by using timer delay or PWM function it produces frequencies to the buzzer, frequencies can be modified by changing the embedded coding.

# Analog Piano

Pulse width modulation controls the on/off period of the logical output that’s generated from the oscillator circuit, Digital modulation of the output voltage is more efficient than analog, this signal sets high(5v) low(0V) at a predefined time and speed, a time during signal stays high is called “one time” is called duty cycle, if signal always one that means 100% duty cycle, a time during signal stays low called “off-time”, if always off that means 0% duty cycle. One cycle on/off period duty cycle, how fast it is finishing determines the frequency. Clock input at PIN2, which determines output frequency, PIN5 variable voltage input, this will decide PWM at the output at PIN3. PIN2 generates corresponding triangle waves at PIN6,7 of the IC, the whole width is determined by “Ra” and C3 capacitor.

Original Values

| Music Tone | C5 | D5 | E5 | F5 | G5 | A5 | B5 | C6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency (Hz) | 523 | 587 | 659 | 698 | 784 | 880 | 988 | 1047 |

Calculated Values

| Music Tone | C5 | D5 | E5 | F5 | G5 | A5 | B5 | C6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency (Hz) | 523 | 586 | 662 | 700 | 784 | 880 | 989 | 1049 |

Bothe values are almost same

- Ton = 0.693( Ra + Rb)C

Ton = 0.693(14k + 6.8k)0.1uF = 1.44ms

- Toff = 0.693(Rb)C

Toff = 0.693(6.8k) 0.1uF = 0.47ms

- Total Time period = Ton + Toff

Total time period = 1.44ms + 0.46ms = 1.9ms

- Freqency = 1/ Total time perild

C5 Feq = 1/1.9ms = 522.82 = 523

C5 = 522.82 = 523

D5 = 586.58 = 586

E5 = 661.92 = 662

F5 = 700.48 = 700

G5 = 784.23 = 784

A5 = 879.87 = 880

B5 = 989.71 = 989

C6 = 1049.45 = 1049

# Digital Piano

A typical microcontroller contains peripheral devices in a single chip that include a processor, memory, and input/output (I / O).

The microcontroller is inserted in the system to control the specific feature of the device. This is achieved by interpreting the data it collects from its I / O peripheral devices using its central processor. The transient data that the microcontroller(MCU) receives is stored in its memory location where the processor accesses it and uses the instructions stored within the program memory to decode and apply the incoming data. It then uses its I / O peripherals to interact and take necessary action. It includes doing simple algebra, logic, and I / O operations. It also executes data transfer operations that relay commands to other elements of the wider embedded system.

Microcontrollers run at a higher speed, for example, 8Mhz or higher speed as defined in the datasheet specifications of MCU, more speed, work can be done faster and efficient, based on the applications

A Delay loop is used to suspend the execution of the program or delaying the next execution of a program for a particular time. There are different ways to generate a delay function cristal oscillator frequency as one the machine cycle, one way of using delay function is by using predefined libraries”<util/delay.h>” in libraries section, using dealy_(ms) can hold up cristal to dealy as the defined time delay. Another way is by using for loop “ for (i=0, I <250, i++);{}, the loop will repeat until the defined number matches. Another method is by using registers selecting timer and flag, timer start from “0” count flag is set with dealy number as we required when the timer reaches the limit and rollover, starts from the “0” same process will be repeated.

When we click a pushbutton or a toggle or a micro switch, two metal sections come into contact to make the supply shorter. But they do not connect instantly, but the metal pieces connect and disconnect multiple times before the final secure connection is made. The same thing happens when the button is pressed. This results in incorrect or multiple triggers like a button being pushed multiple times. Switch bouncing is the non-ideal action of any switch that produces several transitions of a single input.  But it creates issues when dealing with logic or digital circuits. It is then used to eliminate the bounce from the Switch debouncing circuit. Using delays in the program prevents debouncing, but causes the controller to hold for some time, the best way of doing this is using interrupts in the program.

# Practical Session

### Half Wave Bridge Dual Rail Power Supply Project

Describe how the PSU circuit is tested, what equipment is used

- Multimeter as a Current Meter

- Ocsilloscope

- Variable Electronic DC load

Explain the difference between AC and DC coupling in the oscilloscope

setting and how different settings are used to take measurements

Differen between AC and DC coupling in the oscilloscope

- In the oscilloscope when selected DC coupling, I can check DC output of my power supply, ouput of regulator.

- In the oscilloscope when slecged AC coupling, I can check AC ripple of my power supply, output of regulator before suppression of alternating wave form after rectification.

- Vertical devision of the oscilloscope is is voltage reading can chang by rotating the voltage division rotory button.

- Horizental is time division by rotating rotatry button can change the seconds/division.

- By turning torory button ocsilloscople will get static display.

- By selecting measue button can measure DC ripple output values or DC ouput voltage, by selecting cursor one and two can see the vertical value of voltage and horizontal value of the time division within the selected cursion one and two.

Use an oscilloscope to measure and record the RMS voltage and peak-to-peak voltage for the

+12VDC supply.

The measurements should start from no load to 100mA in 10mA step. Observe

and explain the changes to the voltage level. Present the results in table and graph.

| Sl no | Load(mA) | 12 DC(V) | 12V rms(mV) | Load (mA) | 5 DC(V) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.86 | 25.4 | 12.6 | 1.54 | 4.86 |
| 2 | 10.1 | 29.8 | 12.3 | 10 | 4.81 |
| 3 | 19 | 34.7 | 12.1 | 20 | 4.78 |
| 4 | 30 | 38.5 | 12 | 30 | 4.71 |
| 5 | 40 | 43.1 | 11.5 | 40 | 4.66 |
| 6 | 50 | 49.8 | 11.3 | 50 | 4.61 |
| 7 | 60 | 53.7 | 11.2 | 60 | 4.6 |
| 8 | 70 | 58.6 | 11.1 | 70 | 4.58 |
| 9 | 80 | 63.7 | 11.1 | 80 | 4.53 |
| 10 | 90 | 67.1 | 11 | 90 | 4.51 |
| 11 | 100 | 72 | 10.9 | 100 | 4.5 |

### 2. Analog and Digital Piano Project

Use an oscilloscope to measure and record the PWM waveform. Compare the measurements with the calculated values. Complete the measurement for both analog and digital ways. Present the results in tables.

PW Digital graphs

PWM Analog graphs

# Conclusion

The project is designed with an electronic embedded system piano music instrument contains a microcontroller, oscillator, and buzzer, and many other electronic components embedded into the system, to produce music of piano instrument. initially started with power circuit design, created schematic and PCB in Altium designer. The most difficult and fun part of the project is arranging components on PCB by an understanding of the schematic flow of circuit connections. Initially had lots of confusion with setting up the rules for schematic and PCB but eventually, after finishing the 1st project understand what's the real use of rules and why we set up rules. Another most enjoyable part of the project is soldering through-hole components and surface mount components, assembling very minor size digital electronic components using a special soldering machine. 3rd part is coding the microcontroller by suing Atmel studio applying programming knowledge learned from the other papers. After uploading coding to the microcontroller, when we see the piano started working happiest movement of the project. After completing major parts of the piano last part is the understanding theory of the circuit how the electronic parts are designed with engineering knowledge, a bit of calculation of how frequencies, resistance, and capacitors are designed to work piano, measuring square waves and Analog waves for better understanding of the difference between digital and analog circuits, whats makes big changes in both circuits and measuring signals with test instruments. Overall learn a lot from this paper on how electronic devices are designed in the real world

# References

Generate Pulse Width Modulation (PWM) Signal using 555 Timer IC. Circuit Digest. (2020). Retrieved 24 October 2020, from https://circuitdigest.com/electronic-circuits/555-timer-pwm-generator-circuit.

What is a Microcontroller and How Does it Work?. IoT Agenda. (2020). Retrieved 24 October 2020, from https://internetofthingsagenda.techtarget.com/definition/microcontroller.

What is Switch Bouncing and How to prevent it using Debounce Circuit. Circuit Digest. (2020). Retrieved 24 October 2020, from https://circuitdigest.com/electronic-circuits/what-is-switch-bouncing-and-how-to-prevent-it-using-debounce-circuit.